# BEAT — Operational Carbon Savings

Methodology & Calculation Logic — FINAL

Version 1.1 (final — all decisions resolved, verification passed)  •  2026-07-03  •  Rohit Nepali, HEAT GmbH
Codebase: origin/qa (branch work-savings).  Data: Operational_Carbon_Benchmark_restructured.xlsx.  Companion: Operational_Savings_Logic_Companion.xlsx.  SRD: 02.2 System and Business Requirement Document v2.0.pdf.
Interactive UI mockup: https://claude.ai/code/artifact/3d80ec21-08e7-441d-bdc7-1120661c10c0

This document is the complete, final specification of the operational-savings logic. Every design decision is resolved; the numbers below were verified against the source workbook (35/35 derived rows exact; worked example reconciles with the workbook’s own example). It contains no code — implementation follows this spec.

# 1. The two v1 savings layers (+ one deferred)

Operational savings are presented in two independent layers on the Savings tab. They answer different questions and are never summed with each other.

| Layer | Question it answers | Excel sheet(s) | v1 scope |
| --- | --- | --- | --- |
| 1. Whole-building benchmark | How far is the building from its country/type/climate national average and from best practice? | 4_EPI_Benchmarks, 6_India_ECBC_Tiers, 5_Grid_Emission_Factors | IN |
| 3. Optimisation measures + refrigerant | How much does each measure (VSD, DCV, heat recovery, sensors, LED, heat-pump WH, regenerative, VVVF, BMS) save, plus Scope-1 refrigerant transition? | 2_Saving_Measures + refrigerant GWP (in DB) | IN |
| 2. Equipment-efficiency replacement | Savings from replacing equipment with efficient units (kW/RT, W/CMH, LPD, COP deltas). | 1_System_Efficiency, 3_LPD_Benchmarks | Deferred to v2 by decision |

# 2. Data BEAT collects — complete for v1

All inputs required by the v1 logic are already collected by BEAT or derivable from collected fields. Full inventory: companion workbook, Field_Inventory sheet.

Gross floor area (total_floor_area); conditioned area (cond_floor_area) — the Office <50%/>50% AC split is derived as cond/total >= 50%.

Actual annual energy: EnergySummary.total_kwh (auto-summed from the six system models, or manual). Actual EUI = total_kwh / total_floor_area.

Building type AND sub-type from one field: Building.category -> CategorySubcategory (category.name + subcategory.name). The subcategory carries the sub-type signal (hotel star band, office day/24h use, retail sub-type).

Climate: Building.climate_zone -> ClimateType.name (hot-dry, warm-humid, composite, temperate, cold, tropical-wet).

Country: Building.country.name — India / Indonesia / Vietnam / Cambodia / Thailand.

Grid emission factor: derived at runtime from the electricity EPD (B6 GWP / declared_amount); equals the workbook sheet-5 value (India 0.7051).

Measure install-state flags and refrigerant data (type, quantity, GWP, leakage %) on the six building_operation system models.

# 3. How each Excel sheet is used

| Sheet | Provides | Used for |
| --- | --- | --- |
| 0_README | Conventions | Units; % as decimals; blank cell = no benchmark (never synthesised). |
| 1_System_Efficiency | Baseline efficiency per system per country + LEED/EDGE | Layer 2 (v2). |
| 2_Saving_Measures | Measure % per system: VSD 20%, heat recovery 30%, DCV 30%, LED 40%, sensors 20%, heat-pump WH 50%, regenerative 20%, VVVF 40%, BMS 20% | Layer 3 toggle percentages. |
| 3_LPD_Benchmarks | Lighting power density (W/m2) per type per country | Layer 2 (v2). |
| 4_EPI_Benchmarks | National-average EUI (kWh/m2/yr) per type x sub-type x climate x country | Layer 1 national-average line; base for India best practice. |
| 5_Grid_Emission_Factors | kgCO2/kWh per country | EPI -> carbon conversion; reference for the derived grid factor. |
| 6_India_ECBC_Tiers | ECBC+ / Super-ECBC reduction % by climate x India type | India best practice = national EPI x (1 - Super-ECBC%). |

# 4. Taxonomy mapping (verified against seeded data)

BEAT has two category taxonomies — a global set (migration 0002) and an India set (india_category_mapping.json). Both map to the benchmark taxonomy as below; the full table incl. every subcategory is in the companion (Category_Mapping).

| Taxonomy | category.name | subcategory.name | EPI type | EPI sub-type | ECBC best-practice type |
| --- | --- | --- | --- | --- | --- |
| Global | Office | Grade A/B/C | Office | by AC% | Office (Regular Use) |
| Global | Hotels | 1/2/3 star | Hotel | Up to 3-star | Hotel (No Star and Star) |
| Global | Hotels | 4/5 star | Hotel | Above 3-star | Hotel (No Star and Star) |
| Global | Resorts | any | Hotel (proxy, noted) | Up to 3-star | Resort |
| Global | Retail | Shopping Mall | Shopping Mall | - | Shopping Mall |
| Global | Retail | Supermarket | Shopping Mall | - | Supermarket |
| Global | Retail | Department Store / Small Food / Big Box | Shopping Mall | - | Strip retail |
| Global | Healthcare | hospitals (all) | Hospital | - | Hospital |
| Global | Healthcare | Clinics / Diagnostic / Nursing Homes | Hospital | - | Outpatient |
| Global | Education | all | Institute | - | Schools and University |
| Global | Homes / Apartments | any | Residential | - | LEED Residential (non-India rule) |
| Global | Instustrial (sic) / Mixed Use | any | no benchmark -> Layer 1 hidden | - | - |
| India | Office/Business | Day time use (ex Grade A/B) | Office | by AC% | Office (Regular Use) |
| India | Office/Business | 24 hour use (ex Grade C) | Office | by AC% | Office (24 Hours) |
| India | Hospitality | No-star Hotels (ex 1-star) | Hotel | Up to 3-star | Hotel (No Star and Star) |
| India | Hospitality | Star Hotel (ex 2-5 star) | Hotel | Up to 3-star (approximation, noted) | Hotel (No Star and Star) |
| India | Hospitality | Resort | Hotel (proxy, noted) | Up to 3-star | Resort |
| India | Shopping complex | Shopping Mall | Shopping Mall | - | Shopping Mall |
| India | Shopping complex | Stand-alone Retails (ex Dept Store + Big Box) | Shopping Mall | - | Strip retail |
| India | Shopping complex | Super Markets | Shopping Mall | - | Supermarket |
| India | Health care | Hospital | Hospital | - | Hospital |
| India | Health care | Out-patient Healthcare | Hospital | - | Outpatient |
| India | Educational | all | Institute | - | Schools and University |

Climate: hot-dry -> Hot & Dry; warm-humid -> Warm & Humid; composite -> Composite; temperate -> Temperate ("Moderate" in the Office >50% AC rows is the same family); tropical-wet -> Tropical Wet; cold -> Composite (approximation, noted). Country: name matches the EPI column directly. A blank EPI cell means Layer 1 is hidden for that building.

# 5. Layer 1 — whole-building benchmark

actual_EUI        = EnergySummary.total_kwh / total_floor_area          [kWh/m2/yr]
national_EUI      = EPI[type][sub-type][climate][country]               (sheet 4)
best_practice_EUI = India:      national_EUI x (1 - Super_ECBC_pct)     (sheet 6)
                    non-India:  LEED EUI by type (SRD B.1.14):
                                Office 136 | Hotel 157 | Retail/Mall 267 | Hospital 447 | Residential 111
                    GUARD: best practice is shown only when it is BELOW the national average;
                           otherwise the national-average line is shown alone.

grid_EF   = electricity EPD B6 GWP / declared_amount   (== sheet 5)
carbon    = EUI x grid_EF                              [kgCO2eq/m2/yr]
position  = place actual on the scale [best_practice .. national_average]
gap saving (when actual_EUI > best_practice_EUI):
   per year:  (actual_EUI - best_practice_EUI) x grid_EF        [kgCO2eq/m2/yr]
   energy:    (actual_EUI - best_practice_EUI) x GFA            [kWh/yr]
   lifetime:  per-year value x reference_period (default 50)    [kgCO2eq/m2]

Verified example — India, Office, Composite, >50% AC, GFA 1,000 m2, 160,000 kWh/yr, grid 0.7051:

actual EUI 160  -> 112.8 kgCO2eq/m2/yr
national 179    -> 126.2   |   best practice 179 x (1-0.22) = 139.6 -> 98.4
position: 51.8% of the range (better than 48%)
gap: 20.4 kWh/m2/yr = 20,400 kWh/yr = 14.4 kgCO2eq/m2/yr = 719 kgCO2eq/m2 over 50 yr

# 6. Layer 3 — optimisation measures, BMS, refrigerant

Per-system baseline carbon = system annual kWh x grid_EF / GFA (identical to the Systems tab). Each active toggle removes its sheet-2 percentage of the system’s remaining carbon; measures on one system stack multiplicatively; a measure already installed (per the equipment flags) is shown ON and locked with zero additional saving.

| System | Measure | % | Install-state field |
| --- | --- | --- | --- |
| Cooling | VSD on compressors & fans | 20 | chiller.variable_speed_drives |
| Cooling | Heat recovery system | 30 | chiller.heat_recovery_system |
| Ventilation | Demand-controlled ventilation (DCV) | 30 | ventilation.demand_controlled_ventilation |
| Ventilation | VSD on AHU & FCU fans | 20 | ventilation.variable_speed_drives |
| Lighting | LED retrofit on fluorescent zones | 40 | lighting.lighting_bulb_type (non-LED share) |
| Lighting | Occupancy & daylight sensors | 20 | lighting.sensors_installed |
| Hot water | Heat-pump water heater | 50 | hot_water.type_of_hot_water_system |
| Lifts | Regenerative drives | 20 | lift.lift_regenerative_features |
| Lifts | VVVF drive & sleep mode | 40 | lift.vvvf_sleep_mode |
| Whole building | Building Management System (BMS) | 20 | none — always offered as a global toggle |

BMS applies LAST, multiplicatively, to the whole-building electricity total remaining after the per-system measures. It does not apply to the refrigerant (Scope 1) component.

Refrigerant transition (Scope 1): saving = refrigerant_qty_kg x (leakage%/100) x (baseline_GWP - natural_GWP) / GFA, summed over AC + chillers; target = lowest-GWP natural refrigerant (R-290 ~ 3, R-717 ~ 0, R-744 ~ 1). Additive to the electricity result.

Optimized total = sum of optimized systems (x 0.8 if BMS on) + Scope-1 component. Reduction % = saving / baseline total.

# 7. Savings-tab presentation

Interactive mockup (live toggle logic, verified example data): https://claude.ai/code/artifact/3d80ec21-08e7-441d-bdc7-1120661c10c0

Top — Layer 1 benchmark card: gradient scale with three markers (Best Practice | Your Building | National Average, kgCO2eq/m2/yr), tier pill, gap stats in all three units (kgCO2eq/m2/yr, kWh/yr, lifetime kgCO2eq/m2), and a source line naming the benchmark and any approximation.

Left — Carbon reduction card: baseline vs optimized (kgCO2eq/m2/yr), reduction %, and the lifetime pair (x reference_period) beneath; then Savings-by-system with one line per system + Scope 1 + BMS.

Right — Optimization strategies: per-system section (colour-coded, baseline intensity + share badge) with measure rows and toggles; refrigerant row (tagged SCOPE 1) under Cooling; BMS in a Whole-building section at the end. Already-installed measures render ON, locked, labelled "Already installed".

Approximated benchmarks (Star Hotel tier, cold -> composite, Resort proxy) carry a short "approximation" note in the benchmark card’s source line.

Both time bases shown everywhere a headline figure appears: per-year primary, lifetime (x reference_period, default 50 yr) secondary.

Layer 1 and Layer 3 are separate views on the tab; their figures are never added together.

# 8. Decision register (all resolved)

| # | Decision | Resolution |
| --- | --- | --- |
| 1 | Baseline source | Bottom-up per-system energy already collected (EnergySummary); no top-down EPI estimation. |
| 2 | Measure stacking | Multiplicative per system; BMS last on the electricity total. |
| 3 | Already-installed measures | Shown ON and locked, zero additional saving. |
| 4 | Time basis | Per-year primary; lifetime (x reference_period) shown alongside. |
| 5 | v1 scope | Layer 1 + Layer 3 combined on the tab; Layer 2 deferred to v2. |
| 6 | India best practice | National EPI x (1 - Super-ECBC%), sheet 6. |
| 7 | Non-India best practice | LEED whole-building EUI by type (SRD B.1.14), with the guard: shown only where LEED < national EPI (21 combinations fail this and show national-average only). |
| 8 | Hotel tier | Inferred from subcategory; India "Star Hotel" (spans 2-5 star) defaults to Up-to-3-star with an approximation note. |
| 9 | Cold climate | Falls back to the Composite EPI row with an approximation note. |
| 10 | India 24-hour offices | Office EPI + ECBC Office (24 Hours) tier (not BPO). |
| 11 | Department-store retail | ECBC Strip retail in both taxonomies (Department Store, Stand-alone Retails, Non-food Big Box). |
| 12 | BMS | Included as a global 20% toggle, always offered. |
| 13 | Blank benchmark cells | Layer 1 hidden for that building; values are never synthesised. |
| 14 | Refrigerant target | Lowest-GWP natural refrigerant available (R-290 / R-717 / R-744). |

# 9. Verification record

Derived best-practice table (companion, Derived_BestPractice_EUI_IN): 35/35 rows recomputed from sheets 4 & 6 — exact match.

Worked example reconciles with the workbook’s own example (179 x 0.7051 = 126.2).

Taxonomy verified against migration 0002 (global), india_category_mapping.json (India) and migration 0035 (climate).

LEED-vs-national scan: 21 non-India combinations where LEED >= national EPI identified and covered by the decision-7 guard.

Next step: implementation on branch work-savings per this spec (wiring into building_stats.py and savings_tab.html), on user approval.

# 10. Edge-case rules (added after implementation-readiness review)

These rules remove all remaining implementer guesswork. Each is consistent with the decision register (S8).

Benchmark position formula: position% = clamp((actual_carbon - best_practice_carbon) / (national_carbon - best_practice_carbon) x 100, 0..100). Tier labels: actual <= best practice -> "Exceeds best practice"; <= national average -> "Between best practice and national average"; > national average -> "Above national average". When best practice is unavailable (guard or no LEED value), show the national-average marker only and no tier based on the range.

Grid emission factor fallback: primary source is the electricity EPD (B6 GWP / declared_amount). If no electricity EPD is entered (derived factor 0), fall back to the reference value in Country_GridEF (companion / sheet 5) with a note "reference grid factor used". If the country is also unknown, the operational savings view shows the same "grid factor unavailable" warning state as the Systems tab.

Missing conditioned area: if cond_floor_area is empty or zero, the Office AC split defaults to ">50% AC" with an approximation note (typical commercial case).

Already-installed rule: a measure counts as installed only when EVERY record of that system has the flag set; if the system has no equipment records (energy entered manually), the measure is shown available (OFF), since install state is unknown.

LED retrofit with partial LED: if at least one lighting record is non-LED, the measure is available and its 40% applies to the full lighting-system carbon (v1 simplification, consistent with sheet 2 being a system-level percentage; note as approximation). If all records are LED, it is shown installed/locked.

Refrigerant transition: cooling units whose refrigerant GWP is already <= the natural-refrigerant target contribute zero to the Scope-1 saving; units with missing refrigerant data are skipped.

Building types without a LEED value (Institute, BPO) in non-India countries: national-average line only (extension of the decision-7 guard).

LEED Hotel value (157) applies to both hotel tiers (Up to 3-star and Above 3-star).

Missing actuals: if EnergySummary is absent or total_kwh is empty, Layer 1 is hidden (no actual EUI). Layer 3 shows only systems with energy > 0, exactly as the Systems tab does.

Rounding for display: carbon intensities to 1 decimal, percentages to 1 decimal, energy (kWh) to whole numbers; lifetime figures to whole numbers.

Data-source completeness: with this addendum, the methodology document plus the two workbooks are self-sufficient for implementation. The LEED table now also lives in the companion (LEED_BestPractice_NonIN); refrigerant GWPs come from the application database (RefrigerantGWP), seeded per the refrigerant list in the codebase; the SRD remains background reference only.